French Signals for Train Director

This set of signals intends to represent the main signals used in France.

It is, of course, not exhaustive.

For some signals kind, a file with suffix _yb is also given

yb stands for “yellow blinking”

In some cases, when the traffic is heavy, and signals are relatively close, it is necessary to warn the trains sooner.

So instead of to have the 3 current states: read, yellow green, a fourth state is used, so the sequence of a signal is then: red, yellow, yellow blinking, green

A small layout “French_signals.trk” is given to show all kinds of signals that have been created.

sig_for_bloc.tds  and  sig_for_bloc_yb.tds

Use to manage interval between trains

sig_for_scar.tds  and  sig_for_scar_yb.tds

It is a “bloc” signal, with a red light added

Cannot be crossed if 2 reds

Use, for instance, to protect switches and crossing tracks.

sig_for_switch.tds  and  sig_for_switch_yb.tds

This signal is placed before a switch.

When the switch is set to side track, 2 yellow light are blinking

If the switch is set to straight track, the behavior is the same than “sig_for_scar.tds”

sig_for_switch_warn.tds and sig_for_switch_warn_yb.tds

Use to warn that, after the next signal (which can be a “sig_for_switch” for example), the switch is set to the side track.

If the switch after the next signal is set to straight track, the 2 yellow upper lights are not blinking, and the signals acts as a “bloc” signal.

sig_for_switch_warn_car.tds and sig_for_switch_warn_car_yb.tds

Use to warn that, after the next signal (which can be a “sig_for_switch” for example), the switch is set to the side track.

If the switch after the next signal is set to straight track, the 2 yellow upper lights are not blinking, and the signals acts as a “sig_for_scar” signal (must be use in the same conditions of immediate protection).

sig_for_switch_full_warn_car.tds and sig_for_switch_full_warn_car_yb.tds

This signal is both a                         and a

The picture above, extract from the French_signal.trk layout show the both functions it has

Signals at line 28 and 33 are the same, but in 2 different situations.

If a switch just after the signal is set to side track, 2 vertical yellow lights are blinking as signal at line 28.  (note the yellow light at the botton of the signal, showing a red signal just after)

If a switch after the next signal is set to side track, then it acts as a warning signal as signal at line 33, and the 2 horizontal yellow lights are blinking.

sig_for_depot.tds

This signal is purple (stop) or white (upper light)

Used on garage tracks

heurtoir.tds

Name is in French because I didn’t find the English word, so I put a picture !!!!

Thanks for giving me the translation

It is use at the end of a track



To set it:

- Set a signal at the end of the track

- In the “signal properties” box, set “signal is always red” and script “heutoir.tds”

If any problem, feel free to contact me : auger_denis@yahoo.fr

Thanks for using French signals  !!!!